Design and configure the Switch network using cisco packet tracer

## Aim:

To design and simulate a Local Area Network (LAN) using switches in Cisco Packet Tracer and verify end-to-end connectivity between multiple PCs using the ping command.

## Procedure:

- Open Cisco Packet Tracer

- Place Network Devices

  - From End Devices, add required PCs and Laptops to the workspace.

  - From Network Devices → Switches, add two 2950/2960 switches.

- Connect End Devices to Switches

- Connect Switch to Switch

  - Use a Copper Cross-Over cable.

  - Connect FastEthernet port of Switch0 to FastEthernet port of Switch1.

- Assign IP Addresses

  - Click each PC/Laptop → Desktop → IP Configuration.

  - Assign IP addresses in the same network.

  - Example:

    - PC0: 11.10.1.1 / 255.255.255.0

    - PC1: 11.10.1.2 / 255.255.255.0

    - PC2: 11.10.1.3 / 255.255.255.0

  - Leave Default Gateway blank (no router used).

- Check Physical Connections

  - Ensure all cable indicators turn green, showing active links.

- Verify Connectivity

- Observe Results

  - Successful replies indicate proper connectivity.

  - If requests time out:

    - Verify IP addresses

    - Ensure same subnet

    - Check cable types and switch ports

DIAGRAM:

OBSERVATION:

- A LAN was designed using switches in Cisco Packet Tracer.

- Multiple PCs were connected to switches using appropriate cables.

- IP addresses were assigned within the same network.

- No router or default gateway was required.

- Switch-to-switch communication enabled data transfer between PCs.

- Connectivity was tested using the ping command.

- Successful ping replies confirmed proper network connectivity.

RESULT:

The LAN was successfully designed and all connected PCs communicated with each other without errors.